420-421-VA – Fall 2025

Student Name (1): Cyril Efren Fabro  Student Number: 6243515

ASSIGNMENT #2 – Result ___/100%

Five percent (5%) of the final Grade

Submit Your Assignment by October 17th by 23:55, 2025

Instructions:

- Please complete this Assignment in MS Word.

- Once completed, save the document with your Name & Student number

- If you are doing an assignment with a partner, make sure that both names are on the document.

- Upload to Léa section before the deadline mentioned here and in the portal.

Note: Lea system will not accept late submissions.

In this assignment you will learn how to use the design functions of Packet Tracer to create a simple local network. You will also learn how to use the simulation feature of Packet Tracer to observe the impact of your configuration and troubleshoot possible problems.

This lab illustrates the concept of network collision, a problem typical of networks relying on legacy layer 1 technologies.

Objective

STEP 1: CONFIGURATION (30%)

1. Start Packet Tracer

2. Save your work right away in a file called Lab2-your_last_names.

3. Drag four computers and a hub on your network and rename the PCs Computer1 to Computer4.

4. Select the appropriate type of cable to connect the PCs to the hub

5. Assign an IP address to each PC ranging from 192.168.10.2 (PC1) to 192.168.10.5 (PC4).

Don’t forget to assign a subnet mask of 255.255.255.0 to all PC’s

STEP 2: SIMULATION (70%)

Use the simulator to send a ICMP packet (ping) from Computer 1 to Computer 4 using the Capture/Forward button (this button manually activates the simulation one hop at a time) and answer the following questions:

10% - 1. What happens after the packet leaves the hub and why is it happening?

Because we are using a hub, the Computer1 is sending the packet to all the computers that are connected to the hub, and eventually reaching the appropriate Computer (Computer 4)

10% - 2. What happens when the packet reaches Computer 2 and 3?

Both Computer 2 and 3 do not accept the packet as it is not meant for them

10% -3. You saw two types of packets in this simulation. Can you explain why and what these packets are?

Echo Request Packets (The packets that are sent to computer 4 from computer 1)

Echo Reply Packets (The packets that are received to computer 1 as a response from computer 4)

5% -Delete the previous simulation (not the network) and prepare a new one by sending a ping ICMP packet from Computer 1 to Computer 4 and another one from computer 2 to computer 3.

5% -Use the Capture / Forward button to manually activate the simulation and answer the following questions:

10% -4. What happens when the packets reach the hub? (Drag your cursor over the envelope to view some information)

A collision occurs (Fire Icon)

10% 5. Why is this happening?

Because we are using a hub. This is because the hub does not manage collisions or how to solve it

10% 6. How would you fix this problem?

My solution would be to use a router, as it has the capacity to manage and control collisions

INSTRUCTIONS:  Save the completed file with your Student ID and Name (one student name and id is sufficient) and submit it as an attachment Lea.